Calculation of Mass matrix

M=diag(m1,m2,m3,m4), Where mi=ρwVi

Rho ρw

Rho for T1 to T3 same at 983.13 kg/m3 at 60 degrees centigrade

Rho for T4 at 995.71 kg/m3 at 30 degrees centigrade

Volume Vi

V1 Producer Volume

Cylindrical tank

V1=πDin24  Hwater

- Din: inner diameter 60m

- Hwater: actual water height

- fill fraction : 0.9

- Htank : 10m

If the tank total internal height is Htank[m], then:

Hwater=(fill fraction)×Htank

V2 and V4 Pipes

Using inner diameter:

V2,4=πDs,in24  Ls

- Ds,in: inner diameter pipe 0.7m

- Ls: pipe length 20m

V3 Consumer Volume

DHN water is in the tubes which are cylindrical assumed and complete heat exchanger is rectangle assumed

V3=Ntubes  πdin24  Ltube

- Ntubes: number of tubes 20

- din: tube inner diameter 0.0001m2

- Ltube: effective tube length in contact 0.5m

Mass flow rate Kg/s (Mediated Variables)

q(t)=q1,2q2,3q3,4q4,1

Calculation of Heat-loss coefficients: κ=κ1κ2κ3κ4⊤

V1 Producer Volume

Tank surface area A  m2

Side

Aside=πDinHwater

Aside= π(60)(9)

Aside= 540π

Bottom

Abottom=πDin24

Abottom=π(602)4

Abottom=900π

Total internal area

A=Aside+Abottom=(540π+900π)=1440π

Thermal resistances C/W

Insulation (planar approximation):

Rins=tinsλinsA

Insulation thickness: tins=0.10 m

Insulation conductivity: λins= 0.0.35 W/mC

Outside convection:

Rout=1hoA

Outside convection (outdoor air): ho = 10 W/Mm2C

Total:

Rtot=Rins+Rout=C/W
UA = overall conductance W/C

UA=1Rtot

Water heat capacity: cp=4180 J/(kgC)

κ1=UAcp

V2 and V4 Pipes

Insulation resistance per meter Rins'C/W per m

Rins'=ln⁡(r2/r1)2πλins

r1 Inner radius of the insulation layer

r2 outer radius of the insulation layer

If insulation thickness is tins: then r2=r1+tins

Outside convection resistance per meter Rout'

Rout'=1ho 2πr2

Rtot= Rins'+ Rout'

UA = overall conductance W/C

UA=LRtot

κ2,4=UAcp

V3 Consumer Volume

Heat Exchanger outer dimensions are:

- length L= m

- width W= m

- height H= m

Outer surface area A in m2

A=2(LW+LH+WH)
Thermal resistances C/W

Rins=tinsλinsA

Rout=1hoA

Rtot=Rins+Rout=C/W
UA = overall conductance W/C

UA=1Rtot

κ3=UAcp

As consumer node represents radiator it becomes

Radiator heat to room is typically modelled with:

UA≈(hconv+hrad) A

Assumed simple constants:

- Natural convection to air: hconv≈10 W/m2.C

- Radiation “equivalent” coefficient: hrad≈5.5 W/m2.C

So htotal= hconv+hrad W/m2.C

Then

κ3=UAcp=htotalAcp

Ambient Temperature, in Centigrade

Ta(t)=Ta1Ta2Ta3Ta4⊤

Ta(t)=10102010⊤

States, In Centigrade

T(t)=T1T2T3T4⊤

Mediated Variables, Mass flow rate Kg/s

q(t)=q1,2q2,3q3,4q4,1⊤

Mediated Variable, P(t) Input Power of Producer

B=1cp1000

Measured Disturbance Pd(t)

Bd=1cp0010

General Form of ODE

MT(t)=Lκ(q(t)) T(t)+B P(t)+d(t)
d(t)=-Bd Pd(t)+κ Ta(t)