- Topology (4 thermal states, 1 producer, 1 consumer)

Producer, producer supply pipe, consumer and consumer supply pipe towards producer is chosen as vertices.

- v1 = producer volume (where heat is injected)

- v2 = supply pipe volume

- v3 = consumer volume (where heat is extracted)

- v4 = return pipe volume

- Directed edges (heated water transport direction)

- e1: v1 → v2

- e2: v2 → v3

- e3: v3 → v4

- e4: v4 → v1

- Incidence matrix B0

Considering order of edges e1e2e3e4= (1,2),(2,3),(3,4),(4,1). Using the note’s definition (+1 incoming, −1 outgoing):

B0=-100+1+1-1000+1-1000+1-1

- Fundamental cycle matrix F

Spanning tree S={e1,e2,e3}, Chord C={e4}.

In this 4-edge loop, 1 edge is a chord, there is exactly 1 independent cycle which gives 1 chord-flow. So q(t)is “the flow circulating around the loop”.

q(t)∈R
q(t)=q(t)q(t)q(t)q(t)

All q(t) are same because of only one independent cycle

Cycle v1→v2→v3→v4→v1
This gives one fundamental cycle containing all four edges, with all positive entries

F=1111

As per

qe(t)=F⊤q(t)

F⊤=1111

qe(t)=1111q(t)q(t)q(t)q(t)

qe(t)=q(t)q(t)q(t)q(t)

So effectively all edge flows are equal: q1=q2=q3=q4=qc. This automatically gives a balanced strongly connected graph (a property used in the notes). The same flow goes through every segment

- Adjacency A(q)

Edges:

- e1:(1,2)with flow q1,2

- e2:(2,3)with flow q2,3

- e3:(3,4)with flow q3,4

- e4:(4,1)with flow q4,1

So

qe=q1,2q2,3q3,4q4,1

and the adjacency is

A(qe)=000q4,1q1,20000q2,30000q3,40

In a single simple loop with one independent cycle flow,

q1,2=q2,3=q3,4=q4,1=q(t)

So qe=q(t) 1, and then A(qe)becomes the simpler matrix where every nonzero entry is the same q(t).

A(q)=000qq0000q0000q0
Dinq=diag(Aq1)

Dout(q)=diag(A(q)⊤1)

As graph is balanced Doutq=Dinq

Then Din(q)=diag(A(q)⊤1)=qI

A(q)⊤=0q0000q0000qq000

1=[1,1,1,1]⊤

A(q)⊤1=0q0000q0000qq0001111=0⋅1+q⋅1+0⋅1+0⋅10⋅1+0⋅1+q⋅1+0⋅10⋅1+0⋅1+0⋅1+q⋅1q⋅1+0⋅1+0⋅1+0⋅1=qqqq=q1
As A(q)⊤1=q1. So

diag(q1)=diagqqqq=q0000q0000q0000q=qI
So, every node has the same “degree” value qin this ring (because one flow enters/leaves each node with magnitude q).

- Indegree Laplacian

L(q)=Din(q)-A(q)=qI-A(q)

L(q)=q0000q0000q0000q-000qq0000q0000q0

L(q)=q00-q-qq000-qq000-qq

- For Modelling DHN Indegree Laplacian is

Each node loses heat to ambient

-κi(Ti-Ta)=-κiTi+κiTa

In vector/matrix form for all nodes at once:

-diag(κ) T  +  κ Ta

Meanwhile the transport/mixing part in the notes ends up using -L(q)T in the temperature ODE form. So, the terms that multiply T are

(-L(q))T+(-diag(κ))T=(-L(q)-diag(κ))T

Heat-loss enters via

Lκ(q)=-L(q)-diag(κ)

- One producer input P(t), one consumer disturbance Pd(t)

Here m=1 producer at node 1, d=1 consumer at node 3. From the notes, the injection/extraction mapping uses 1/cp:

B=1cp1000,Bd=1cp0010

- P(t)≥0: heat injected by producer

- Pd(t)≥0: heat extracted by consumer

- Final ODE model

States, T(t)=T1T2T3T4⊤

Heat-loss coefficients: κ=κ1κ2κ3κ4⊤, ambient temperature Ta

Let M=diag(m1,m2,m3,m4). Where mi=ρwVi, The model is

MT(t)=Lκ(q(t)) T(t)+B P(t)+d(t)
d(t)=-Bd Pd(t)+κ Ta(t)

DHN ODEs

m1T1=q1,2(T4-T1)  -κ1(T1-Ta1)  +1cpP
m2T2=q2,3(T1-T2)  -κ2(T2-Ta2)
m3T3=q3,4(T2-T3)  -κ3(T3-Ta3)  -1cpPd
m4T4=q4,1(T3-T4)  -κ4(T4-Ta4)

- Port-Hamiltonian (pH) formulation

DHN ODE

MT(t)=Lκ(q(t)) T(t)+B P(t)+d(t)
d(t)=-Bd Pd(t)+κ Ta(t)

with

Lκ(q)=-L(q)-diag(κ),d(t)=-BdPd(t)+κ Ta(t).

So, substitute both into the first equation:

MT=(-L(q)-diag(κ))T  +  BP  +  (-BdPd+κTa).

Group terms:

MT=- L(q)T-diag(κ)T+BP-BdPd+κTa.

Define the “temperature deviation” from ambient

Define:

θ=T-Ta1

This means:

T=θ+Ta1

Now differentiate both sides:

T=θ+Ta1

Notes assumption: Ta=0

Ta=0⇒T=θ

So, left hand term becomes

MT=Mθ

Substitute T=θ+Ta1into the right-hand side in the first transport term

-L(q)T=-L(q)(θ+Ta1)=-L(q)θ-Ta L(q)1

L(q)1=0

- L(q)=qI-A(q)

- A(q)1=q1(each row has exactly one outgoing q

- so L(q)1=q1-q1=0

Therefore:

-L(q)T=-L(q)θ

So, the ambient part drops out of the transport term.

Now the loss term:

-diag(κ)T=-diag(κ)(θ+Ta1)=-diag(κ)θ-diag(κ) Ta1

But diag(κ) Ta1=κTa as per notes. So,

-diag(κ)T=-diag(κ)θ-κTa
Substituting in Main Equation

Left:

MT=Mθ

Right:

+κTa cancels -κTa in right term which leaves

-L(q)θ-diag(κ)θ+BP-BdPd

So, the deviation model is:

Mθ=(-L(q)-diag(κ))θ+BP-BdPd

- (pH) state x=cpMθ also called as energy-scaled state. θ and Mdefined above

x(t):=cpMθ(t)∈R4

Thermal energy in a lumped water volume is:

Ei=micp(Ti-Ta)=micpθi

It is energy content (above ambient) stored in each node

xi=micpθi

- Hamiltonian H(x)=12x⊤(cpM)-1x, is the stored energy function.

If the true stored energy is imicpθi22, then Choose a quadratic Hamiltonian (a standard pH storage function consistent with “total energy” as a Lyapunov/storage function)

H=12θ⊤cpMθ

From x=cpMθwe get

θ=(cpM)-1x

Plug into H=12θ⊤cpMθ

H(x)=12((cpM)-1x)⊤(cpM)((cpM)-1x)

cpMcancels one inverse leaving

H(x)=12x⊤(cpM)-1x
which makes both identical

H(x)=12θ⊤cpMθ  =  12x⊤(cpM)-1x

- e:=∇H(x)=(cpM)-1x=θ , effort e is the conjugate variable

Compute gradient:

Hx=12x⊤Qx , with Q=(cpM)-1

For a quadratic function, the gradient is:

∇H(x)=Qx=(cpM)-1x

But we already know:

θ=(cpM)-1x

So,

e=∇H(x)=θ=T-Ta1
which gives

- xi=micp(Ti-Ta) energy stored

- ei=Ti-Ta temperature level

- Putting the dynamics into pH form

Deviation model (ambient removed):

Mθ=(-L(q)-diag(κ))θ+BP-BdPd

We already know

x=cpMθ⇒x=cpMθ

(Reason: Mand cpare constants.)

e=∇H(x)
 e=θ

Now multiply the deviation model by cp

cpMθ=cp(-L(q)-diag(κ))θ+cpBP-cpBdPd

Replace cpMθby xand θby e

x=cp(-L(q)-diag(κ))e+cpB P-cpBd Pd

- The J/ Rdecomposition

Port-Hamiltonian standard structure is:

x=(J-R) e+Gu+Gdw

- J⊤=-J(skew-symmetric) power-conserving interconnection

- R⊤=R⪰0(symmetric Positive Semidefinite) dissipation

The matrix multiplying e is cp(-L(q)-diag(κ)). Splitting -cpL(q)into skew + symmetric parts

Skew part (J(q))

J(q)=12(-cpL(q)-(-cpL(q))⊤)

J(q)=cp2(L(q)⊤-L(q))

L(q)=q00-q-qq000-qq000-qq

L(q)⊤=q-q000q-q000q-q-q00q
J(q)=cp20-q0qq0-q00q0-q-q0q0
This guarantees J(q)⊤=-J(q)

Symmetric part ( -Rflow(q))

-Rflow(q)=12(-cpL(q)+(-cpL(q))⊤)

-Rflow(q)=-cp2(L(q)+L(q)⊤)

Rflow(q):=cp2(L(q)+L(q)⊤)
Rflow(q)=cp22q-q0-q-q2q-q00-q2q-q-q0-q2q

which is symmetric by construction. Then -cpL(q)=J(q)-Rflow(q) only computed for understanding.

-cpL(q)=cp-q00qq-q000q-q000q-q

The heat loss part in matrix is -cpdiag(κ). This is already symmetric and (for κi≥0) positive semidefinite. Then it can be defined as Rloss:=cp diag(κ)

So

-cpdiag(κ)=-cpκ10000κ20000κ30000κ4

-cpdiag(κ)=-Rloss

cpdiag(κ)=Rloss

Rloss=cpκ10000κ20000κ30000κ4

Combining dissipation terms

R(q)=Rflow(q)+Rloss

R(q)=cpq+κ1-q20-q2-q2q+κ2-q200-q2q+κ3-q2-q20-q2q+κ4

Now the whole matrix multiplying e becomes:

-cpL(q)-cpdiag(κ)=(J(q)-Rflow(q))-Rloss

-cpL(q)-cpdiag(κ)=J(q)-R(q)

So

x=(J(q)-R(q))e+Gu+Gdw

Finally, port-Hamiltonian system is:

x(t)=(J(q(t))-R(q(t))) e(t)  +  G u(t)  +  Gd w(t)

with

- e(t)=∇H(x(t)), or e=θ

- input u(t)=P(t),

- disturbance w(t)=Pd(t),

- G=cpB, Gd=-cpBd

- G,Gdfor 1-producer / 1-consumer

We already know

B=1cp1000, Bd=1cp0010

So

G=cpB=1000, Gd=-cpBd=-0010

A standard co-located output is

y(t)=G⊤e(t)=(cpB)⊤(T(t)-Ta1).
producer at node 1

B=1cp1000

So

G=cpB=1000

Now computing

y=G⊤e=1000e1e2e3e4=e1

But e=θ=T-Ta1, hence:

e1=T1-Ta

y(t)=T1(t)-Ta